# Day 1 — Verilog Design + Testbench

Today’s goal:

- Understand module structure

- Learn input/output

- Learn assign

- Write first design

- Write first testbench

- Run simulation

- View waveform

Today we build:

- AND Gate

- OR Gate

# 1. What is Verilog?

Verilog is a Hardware Description Language (HDL).

We use it to describe digital circuits like:

- Gates

- Adders

- Flip-flops

- Processors

Think like this:

| Software | Hardware |
| --- | --- |
| C Program | Verilog Module |
| Function | Circuit |
| Output after execution | Output after signal change |

# 2. Basic Module Structure

module module_name(

input a,

input b,

output y

);

// logic

endmodule

### Important Keywords

| Keyword | Meaning |
| --- | --- |
| module | Start of design |
| input | Input signal |
| output | Output signal |
| assign | Continuous assignment |
| endmodule | End of module |

# 3. AND Gate Design

## Truth Table

| A | B | Y |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 0 |
| 1 | 0 | 0 |
| 1 | 1 | 1 |

# 4. AND Gate Verilog Code

Create file:

and_gate.v

Code:

module and_gate(

input a,

input b,

output y

);

assign y = a & b;

endmodule

# 5. Understanding assign

assign y = a & b;

Meaning:

- Continuously monitor inputs

- If a or b changes

- y updates automatically

& means AND operation.

# 6. Testbench Introduction

A testbench checks whether the design works correctly.

Think:

- Design = Student

- Testbench = Examiner

The testbench gives inputs and checks outputs.

# 7. AND Gate Testbench

Create file:

and_gate_tb.v

Code:

`timescale 1ns/1ps

module and_gate_tb;

reg a, b;

wire y;

and_gate uut(

.a(a),

.b(b),

.y(y)

);

initial begin

$dumpfile("dump.vcd");

$dumpvars(0, and_gate_tb);

a = 0; b = 0;

#10;

a = 0; b = 1;

#10;

a = 1; b = 0;

#10;

a = 1; b = 1;

#10;

$finish;

end

endmodule

# 8. Important Testbench Concepts

## reg

reg a, b;

Used for:

- Storing values

- Giving inputs in testbench

## wire

wire y;

Used for:

- Connecting outputs

## UUT

and_gate uut(

UUT means: Unit Under Test

The actual design module.

## Delay

#10;

Wait for 10 ns before next input.

## dumpfile + dumpvars

$dumpfile("dump.vcd");

$dumpvars(0, and_gate_tb);

Creates waveform file for GTKWave.

# 9. Compile and Run

Open terminal in folder.

## Compile

iverilog -o sim and_gate.v and_gate_tb.v

## Run

vvp sim

## Open GTKWave

gtkwave dump.vcd

# 10. Expected Waveform

| Time | A | B | Y |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 10 | 0 | 1 | 0 |
| 20 | 1 | 0 | 0 |
| 30 | 1 | 1 | 1 |

# 11. OR Gate Challenge (Do Yourself)

## Truth Table

| A | B | Y |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 1 |
| 1 | 0 | 1 |
| 1 | 1 | 1 |

## Task

Create:

- or_gate.v

- or_gate_tb.v

Hint:

assign y = a | b;

# 12. Interview Questions

## Q1. Difference between reg and wire?

| reg | wire |
| --- | --- |
| Holds value | Connection |
| Used in procedural blocks | Used for continuous assignment |

## Q2. Why use testbench?

To verify design functionality before hardware implementation.

## Q3. What is UUT?

Unit Under Test.

# 13. Mini Challenge

Modify AND gate to:

- 3-input AND gate

Inputs:

a, b, c

Expected logic:

y = a & b & c;

# Day 1 Homework

Complete:

- AND gate

- OR gate

- 3-input AND gate

- Run waveform

- Observe signal changes carefully

Do NOT move to Day 2 until: ✅ You can write module syntax without seeing notes
✅ You understand reg vs wire
✅ You can run simulation yourself
✅ GTKWave works properly