Day 23 — File Separation & Clean Coding Style

Signal, today is about writing code like a professional RTL/DV engineer.

Many students can write Verilog.

Few students write clean, maintainable, industry-style Verilog.

Interviewers notice this.

Why File Separation Matters

Bad Practice:

Project/

Adder.v

Adder_tb.v

Counter.v

Counter_tb.v

Mux.v

Mux_tb.v

Decoder.v

Decoder_tb.v

...

After 20 projects:

50+ files

Hard to manage.

Industry Style Project Structure

Example:

Traffic_light_project/

├── rtl/

│   └── traffic_light.v

│

├── tb/

│   └── traffic_light_tb.v

│

├── sim/

│   └── traffic.vcd

│

├── docs/

│   └── README.md

│

└── run.bat

Folder Meaning

Rtl/

Contains:

Design Files

Examples:

Counter.v

Adder.v

Fsm.v

Uart_tx.v

Tb/

Contains:

Testbench Files

Examples:

Counter_tb.v

Adder_tb.v

Fsm_tb.v

Sim/

Contains:

Simulation Outputs

Examples:

Wave.vcd

Log.txt

Docs/

Contains:

README

Project Notes

Diagrams

Naming Convention

Bad:

Module abc;

Good:

Module traffic_light_controller;

Bad:

Reg x;

Reg y;

Good:

Reg clk;

Reg rst;

Reg enable;

Meaningful names improve debugging.

Example Project

RTL File

Counter.v

Module counter(

Input clk,

Input rst,

Output reg [3:0] count

);

Always @(posedge clk or posedge rst)

Begin

If(rst)

Count <= 0;

Else

Count <= count + 1;

End

Endmodule

Testbench File

Counter_tb.v

Module counter_tb;

Reg clk;

Reg rst;

Wire [3:0] count;

Counter uut(

.clk(clk),

.rst(rst),

.count(count)

);

Always #5 clk = ~clk;

Initial

Begin

Clk = 0;

Rst = 1;

#10 rst = 0;

#100 $finish;

End

Initial

Begin

$dumpfile(“counter.vcd”);

$dumpvars(0,counter_tb);

End

Endmodule

Compile

Iverilog -o sim counter.v counter_tb.v

Run

.\sim

Expected Output

Counter values:

0000

0001

0010

0011

0100

...

GTKWave Signals

Observe:

Clk

Rst

Count

Clean Coding Guidelines

Rule 1

One Module Per File

Good:

Counter.v

Contains:

Module counter

Only.

Rule 2

Use Proper Indentation

Bad:

always@(posedge clk)

begin

if(rst)

count<=0;

end

Good:

Always @(posedge clk)

Begin

If(rst)

Count <= 0;

End

Rule 3

Add Comments

Example:

//

// State Register

//

Always @(posedge clk)

Rule 4

Meaningful Signal Names

Bad:

Reg a;

Reg b;

Good:

Reg clk;

Reg rst;

Rule 5

Separate Logic Blocks

FSM Example:

State Register

Next State Logic

Output Logic

Keep separate.

Common Coding Mistakes

Bug 1

Everything in one file

Bad:

Counter

Counter_tb

Adder

Adder_tb

Inside same file.

Bug 2

No Comments

Future debugging becomes difficult.

Bug 3

Poor Naming

Bad:

Reg x;

Good:

Reg enable;

Interview Questions

- Why Separate RTL and Testbench?

RTL:

Design

Testbench:

Verification

Different purposes.

- Why Use Meaningful Names?

Makes debugging easier.

- What Should Be Inside rtl Folder?

Design files only

- What Should Be Inside tb Folder?

Testbench files only

- Why Add Comments?

Improves maintainability and readability.

Challenge 1

Organize Existing Projects

Create:

VerilogPractice/

├── AND_GATE/

│   ├── rtl/

│   ├── tb/

│

├── FULL_ADDER/

│   ├── rtl/

│   ├── tb/

│

├── COUNTER/

│   ├── rtl/

│   ├── tb/

Expected Result

Easy navigation and project management.

GTKWave Signals

Use project-specific signals.

Challenge 2

Add Header Comments

Example:

//--------------------------------------------------

// Module Name : Counter

// Author      : Signal

// Description : 4-bit Synchronous Counter

//--------------------------------------------------

Add to all projects.

Expected Result

Professional-looking code.

Challenge 3

Rename Poor Signals

Change:

Reg a;

Reg b;

Reg c;

To:

Reg clk;

Reg rst;

Reg enable;

Expected Result

Easier waveform debugging.

DV Engineer Observation Checklist

Verify:

✅ RTL and TB separated

✅ Clean folder structure

✅ Meaningful names

✅ Proper comments

✅ One module per file

✅ Easy-to-read code

Professional GitHub Structure

For future projects:

Project_Name/

├── rtl/

├── tb/

├── sim/

├── docs/

├── README.md

└── waveform.png

This looks much more professional to recruiters and interviewers.

Tomorrow — Day 24

Testbench Automation

You’ll learn:

✅ Automatic test generation

✅ Loops in testbench

✅ Exhaustive testing

✅ Self-checking testbench basics

✅ Faster verification techniques

This is where you start thinking more like a Design Verification Engineer than a design engineer.