Day 27 — Interview Coding Practice

Signal, today’s goal is simple:

Can you write Verilog code without looking at notes?

This is exactly what happens in many DV/RTL fresher interviews.

Interview Challenge 1 — AND Gate

Question

Write a Verilog module for a 2-input AND gate.

Solution

Module and_gate(

Input a,

Input b,

Output y

);

Assign y = a & b;

Endmodule

Test Cases

A	b	y

0	0	0

0	1	0

1	0	0

1	1	1

GTKWave Signals

A

B

Y

Interview Challenge 2 — 2:1 MUX

Question

Design a 2:1 Multiplexer.

Solution

Module mux2x1(

Input a,

Input b,

Input sel,

Output y

);

Assign y = sel ? b : a;

Endmodule

Expected Output

Sel	y

0	a

1	b

GTKWave Signals

A

B

Sel

Y

Interview Challenge 3 — Full Adder

Question

Write a Full Adder.

Solution

Module full_adder(

Input a,

Input b,

Input cin,

Output sum,

Output carry

);

Assign {carry,sum} = a + b + cin;

Endmodule

Expected Output

A	b	cin	sum	carry

0	0	0	0	0

0	0	1	1	0

0	1	0	1	0

0	1	1	0	1

1	0	0	1	0

1	0	1	0	1

1	1	0	0	1

1	1	1	1	1

GTKWave Signals

A

B

Cin

Sum

Carry

Interview Challenge 4 — D Flip-Flop

Question

Write a positive-edge-triggered D Flip-Flop.

Solution

Module dff(

Input clk,

Input d,

Output reg q

);

Always @(posedge clk)

Begin

Q <= d;

End

Endmodule

Expected Output

At every positive clock edge

Q = d

GTKWave Signals

Clk

D

Q

Interview Challenge 5 — 4-Bit Counter

Question

Design a synchronous up counter.

Solution

Module counter(

Input clk,

Input rst,

Output reg [3:0] count

);

Always @(posedge clk or posedge rst)

Begin

If(rst)

Count <= 0;

Else

Count <= count + 1;

End

Endmodule

Expected Output

0000

0001

0010

0011

0100

...

GTKWave Signals

Clk

Rst

Count

Interview Challenge 6 — Sequence Detector

Question

Detect sequence:

1011

Expected FSM States

S0

S1

S2

S3

S4

Output

Detected = 1

When sequence is found.

GTKWave Signals

Clk

Din

State

Detected

Frequently Asked Interview Theory Questions

- Difference Between Wire and Reg?

Wire

Wire y;

Assign y = a & b;

Used for continuous assignments.

Reg

Reg q;

Always @(posedge clk)

Begin

Q <= d;

End

Used inside always blocks.

- Difference Between = and <= ?

Blocking

=

Executes immediately.

Non-Blocking

<=

Updates at end of time step.

Use:

Combinational Logic → =

Sequential Logic → <=

- Difference Between Moore and Mealy FSM?

Moore

Output depends on State

Mealy

Output depends on State + Input

- Why Use Reset?

Initialize circuit to known state.

- What is a Testbench?

A module used to verify DUT behavior.

No inputs or outputs.

Common Interview Mistakes

Mistake 1

Using “=” inside flip-flops.

Wrong:

Q = d;

Correct:

Q <= d;

Mistake 2

Forgetting reset.

If(rst)

Count <= 0;

Mistake 3

Missing default in case statement.

Default:

Next_state = S0;

Mistake 4

Using reg for DUT output incorrectly.

Wrong TB:

Reg y;

Correct:

Wire y;

Mock Coding Round

Set a timer for 20 minutes and write from memory:

Task 1

AND Gate

Task 2

2:1 MUX

Task 3

Full Adder

Task 4

D Flip-Flop

Task 5

4-bit Counter

Without looking at notes.

Challenge 1

Create a 4:1 MUX

Inputs

A,b,c,d

Sel[1:0]

Output

Y

Expected Output

Sel=00 → a

Sel=01 → b

Sel=10 → c

Sel=11 → d

GTKWave Signals

A

B

C

D

Sel

Y

Challenge 2

Create a 4-bit Comparator

Outputs:

Gt

Lt

Eq

Expected Output

A>B → gt=1

A<B → lt=1

A==B → eq=1

GTKWave Signals

A[3:0]

B[3:0]

Gt

Lt

Eq

Challenge 3

Create a Mod-10 Counter

Counts:

0

1

2

3

4

5

6

7

8

9

0

1...

Expected Output

0000

0001

...

1001

0000

GTKWave Signals

Clk

Rst

Count

DV Engineer Checklist

Before any interview, make sure you can write from memory:

✅ AND Gate

✅ OR Gate

✅ XOR Gate

✅ Half Adder

✅ Full Adder

✅ MUX

✅ D Flip-Flop

✅ Counter

✅ FSM Basics

✅ Testbench

✅ $display

✅ $monitor

✅ Task

✅ Function

✅ Random Testing

✅ Checker

Tomorrow — Day 28

Mini Project: UART Transmitter

You’ll learn:

✅ UART basics

✅ Serial communication

✅ UART TX design

✅ UART TX testbench

✅ Waveform analysis

✅ Interview questions

✅ GitHub project structure

This is one of the most common protocol-related projects discussed in VLSI interviews.