Day 29 — Mock Design Verification Interview

Signal, today is your mock interview day.

Rules:

✅ No notes

✅ No ChatGPT help initially

✅ Try answering yourself first

✅ Time yourself (45–60 minutes)

After answering, compare with the solutions.

Round 1 — Basic Verilog Questions

Q1

Difference between:

Wire

And

Reg

Q2

Difference between:

=

And

<=

Q3

What is a Testbench?

Q4

What is the purpose of Reset?

Q5

What is the difference between:

$display

And

$monitor

Round 2 — Intermediate Questions

Q6

What is a Flip-Flop?

Q7

What is the difference between:

Combinational Logic

And

Sequential Logic

Q8

Why do we use FSM?

Q9

What is Moore FSM?

Q10

What is Mealy FSM?

Round 3 — DV-Oriented Questions

Q11

What is a Checker?

Q12

What is a Self-Checking Testbench?

Q13

Why do we use Random Testing?

Q14

What is Exhaustive Testing?

Q15

What is a Scoreboard?

Coding Round

Problem 1

Write a 2-input AND gate.

Expected Output

00 → 0

01 → 0

10 → 0

11 → 1

Problem 2

Write a Full Adder.

Expected Output

111

Sum = 1

Carry = 1

Problem 3

Write a D Flip-Flop.

Requirements:

Positive Edge Triggered

Problem 4

Write a 4-bit Counter with Reset.

Expected:

0000

0001

0010

...

Problem 5

Write a 2:1 MUX.

Expected:

Sel=0 → a

Sel=1 → b

Debugging Round

Interviewers often show buggy code.

Find the error.

Debug Question 1

Always @(posedge clk)

Begin

Q = d;

End

What’s Wrong?

Answer

Q <= d;

Sequential logic should use:

<=

Debug Question 2

Module and_gate(

Input a,

Input b,

Output y

);

Assign y = a | b;

Endmodule

What’s Wrong?

Expected:

AND Gate

But code implements:

OR Gate

Correct:

Assign y = a & b;

Testbench Round

Question

Write a testbench for:

Module xor_gate(

Input a,

Input b,

Output y

);

Requirements:

Apply all 4 combinations

Display output

Generate VCD file

Expected Testbench

Module xor_tb;

Reg a;

Reg b;

Wire y;

Xor_gate uut(

.a(a),

.b(b),

.y(y)

);

Initial

Begin

$dumpfile(“xor.vcd”);

$dumpvars(0,xor_tb);

A=0; b=0; #10;

A=0; b=1; #10;

A=1; b=0; #10;

A=1; b=1; #10;

$finish;

End

Initial

Begin

$monitor(

“A=%b B=%b Y=%b”,

A,b,y

);

End

Endmodule

Waveform Questions

Suppose:

Always @(posedge clk)

Begin

Q <= d;

End

Waveform:

Clk ↑

D = 1

Q = ?

Answer

Q = 1

At the positive edge.

GTKWave Checklist

During interview projects, always inspect:

Clk

Rst

Inputs

Outputs

State

Counter

Enable

Self-Evaluation Sheet

Give yourself:

Section	Marks

Theory (15 Questions)	15

Coding (5 Problems)	25

Debugging (2 Problems)	10

Testbench	10

Total:

60 Marks

Evaluation

50–60

Interview Ready

40–49

Very Good

Need More Practice

30–39

Good Foundation

Revise FSM & Testbench

Below 30

Repeat Days 15–29

Bonus Challenge

Without notes, write:

✅ Full Adder

✅ D Flip-Flop

✅ Counter

✅ Sequence Detector

✅ UART TX

If you can write all 5 from memory, you’re approaching fresher DV interview level.

Tomorrow — Day 30 (Final Challenge)

You’ll build a complete project:

ALU (Arithmetic Logic Unit)

Operations:

ADD

SUB

AND

OR

XOR

Including:

✅ RTL Design

✅ Testbench

✅ Self-Checking Verification

✅ GTKWave Analysis

✅ Interview Questions

✅ GitHub Project Structure

This will be the final project of your 30-Day Verilog + Testbench Challenge.