Day 30 — Final Challenge: ALU (Arithmetic Logic Unit)

Signal, congratulations!

You’ve completed 29 days of Verilog practice. Today you’ll build a complete mini-project that combines:

✅ RTL Design

✅ Testbench

✅ Self-Checking Verification

✅ GTKWave Analysis

✅ Interview Concepts

This is the type of project you can put on GitHub and discuss in interviews.

What is an ALU?

ALU stands for:

Arithmetic Logic Unit

It performs mathematical and logical operations.

Used in:

CPUs

Microprocessors

Microcontrollers

DSPs

Operations

Sel	Operation

000	ADD

001	SUB

010	AND

011	OR

100	XOR

RTL Design

Alu.v

Module alu(

Input [3:0] a,

Input [3:0] b,

Input [2:0] sel,

Output reg [4:0] result

);

Always @(*)

Begin

Case(sel)

3’b000: result = a + b;

3’b001: result = a – b;

3’b010: result = a & b;

3’b011: result = a | b;

3’b100: result = a ^ b;

Default: result = 0;

Endcase

End

Endmodule

Testbench

Alu_tb.v

Module alu_tb;

Reg [3:0] a;

Reg [3:0] b;

Reg [2:0] sel;

Wire [4:0] result;

Reg [4:0] expected;

Alu uut(

.a(a),

.b(b),

.sel(sel),

.result(result)

);

Task check_result;

Begin

If(result == expected)

$display(

“PASS A=%d B=%d SEL=%b RESULT=%d”,

A,b,sel,result

);

Else

$display(

“FAIL A=%d B=%d SEL=%b RESULT=%d EXPECTED=%d”,

A,b,sel,result,expected

);

End

Endtask

Initial

Begin

$dumpfile(“alu.vcd”);

$dumpvars(0,alu_tb);

// ADD

A=5;

B=3;

Sel=3’b000;

Expected=8;

#10;

Check_result;

// SUB

A=9;

B=2;

Sel=3’b001;

Expected=7;

#10;

Check_result;

// AND

A=12;

B=10;

Sel=3’b010;

Expected=(12 & 10);

#10;

Check_result;

// OR

A=12;

B=10;

Sel=3’b011;

Expected=(12 | 10);

#10;

Check_result;

// XOR

A=12;

B=10;

Sel=3’b100;

Expected=(12 ^ 10);

#10;

Check_result;

$finish;

End

Endmodule

Expected Output

PASS A=5 B=3 SEL=000 RESULT=8

PASS A=9 B=2 SEL=001 RESULT=7

PASS A=12 B=10 SEL=010 RESULT=8

PASS A=12 B=10 SEL=011 RESULT=14

PASS A=12 B=10 SEL=100 RESULT=6

GTKWave Signals

Add these signals:

A[3:0]

B[3:0]

Sel[2:0]

Result[4:0]

Expected[4:0]

GTKWave Analysis

ADD

A=5

B=3

Result=8

SUB

A=9

B=2

Result=7

AND

1100

1010

1000

Result:

8

OR

1100

1010

1110

Result:

14

XOR

1100

1010

0110

Result:

6

Compile

Iverilog -o sim alu.v alu_tb.v

Run

.\sim

Common Bugs

Bug 1

Wrong Case Value

Wrong:

3’b101: result = a ^ b;

Correct:

3’b100

Bug 2

Missing Default

Wrong:

Case(sel)

...

Endcase

Correct:

Default: result = 0;

Bug 3

Using Wire Instead of Reg

Wrong:

Output [4:0] result;

Correct:

Output reg [4:0] result;

Because result is assigned inside an always block.

Interview Questions

- What is an ALU?

A hardware block that performs arithmetic and logical operations.

- Why is ALU important?

It is the computational core of a processor.

- Why use case statement in ALU?

To select different operations based on control signals.

- Why use always @(*)?

To model combinational logic.

- Why is result declared as reg?

Because it is assigned inside an always block.

Bonus Challenge 1

Add:

101 → NAND

110 → NOR

111 → XNOR

Update RTL and Testbench.

Bonus Challenge 2

Make ALU Parameterized

Parameter WIDTH = 8;

Convert:

[3:0]

To:

[WIDTH-1:0]

Bonus Challenge 3

Random Verification

Run:

For(i=0;i<100;i=i+1)

Generate:

A = $random;

B = $random;

Sel = $random % 5;

Add self-checking logic.

Expected:

PASS = 100

FAIL = 0

What You’ve Learned in 30 Days

Combinational Logic

✅ Gates

✅ Half Adder

✅ Full Adder

✅ MUX

✅ Decoder

✅ Encoder

✅ Comparator

Sequential Logic

✅ D Flip-Flop

✅ JK Flip-Flop

✅ T Flip-Flop

✅ Shift Register

✅ Counter

Intermediate Verilog

✅ Parameterized Modules

✅ FSM

✅ Sequence Detector

✅ Traffic Light Controller

Verification Skills

✅ Testbench Writing

✅ Tasks

✅ Functions

✅ Automation

✅ Random Testing

✅ Checkers

✅ Monitor

✅ Self-Checking Testbench

Projects

✅ Traffic Light Controller

✅ UART Transmitter

✅ ALU

Next Roadmap (DV Engineer Path)

Week 5–6

✅ SystemVerilog Basics

Logic

Always_comb

Always_ff

Arrays

Queues

Mailboxes

Week 7–8

✅ Constrained Randomization

Week 9

✅ Assertions (SVA)

Week 10

✅ Functional Coverage

Week 11–12

✅ UVM Basics

Week 13+

✅ Protocols

UART

SPI

I2C

APB

AXI

At this point you’ll be moving from Verilog learner to Design Verification Engineer candidate.