Day 14 — Revision + Debugging Day

Today is one of the MOST important days for becoming a DV Engineer.

Why?

Because real DV work is mostly:

Finding bugs

Debugging waveforms

Understanding simulation behavior

Fixing RTL/testbench mistakes

Not just writing code.

Today’s Goal

You will learn:

✅ How to debug Verilog code

✅ How to read waveforms

✅ Common beginner mistakes

✅ How interview debugging questions work

✅ How simulation errors happen

Debugging Rule

Always debug in this order:

1. Compilation Errors

2. Simulation Errors

3. Logic Errors

4. Waveform Verification

Part 1 — Common Compilation Errors

Error 1 — Missing Semicolon

Wrong Code

Module and_gate(

Input a,

Input b,

Output y

)

Assign y = a & b;

Endmodule

Problem

Missing semicolon:

);

Correct Code

Module and_gate(

Input a,

Input b,

Output y

);

Assign y = a & b;

Endmodule

Error 2 — Wrong Signal Type

Wrong

Reg y;

Assign y = a & b;

Why Error?

Assign works with:

Wire

Correct

Wire y;

Error 3 — Module Name Mismatch

Design File

Module adder(

Testbench

Half_adder uut(

Problem

Module names must match.

Part 2 — Simulation Errors

Error 4 — No Clock Generation

Wrong Testbench

Initial

Begin

Clk = 0;

End

Problem

Clock never changes.

Counter/DFF will never work.

Correct

Always #5 clk = ~clk;

Error 5 — Forgot Initial Values

Wrong

Reg a;

Reg b;

Problem

Signals become:

X

Unknown state.

Correct

Initial

Begin

A = 0;

B = 0;

End

Part 3 — Logic Errors

These are VERY important.

Compilation succeeds.

Simulation runs.

BUT output is wrong.

Error 6 — Blocking vs Non-Blocking

Wrong Sequential Logic

Always @(posedge clk)

Begin

Q = d;

End

Correct

Always @(posedge clk)

Begin

Q <= d;

End

Rule

Use	Operator

Combinational	=

Sequential	<=

Error 7 — Wrong Sensitivity List

Wrong

Always @(a)

Problem

If b changes:

Block will NOT execute

Correct

Always @(*)

OR

Always @(a or b)

Part 4 — Waveform Debugging

This is the HEART of DV.

Example Counter Debug

Suppose waveform shows:

0000

0001

0010

0010

0011

Problem: Counter repeated value.

Possible reasons:

Clock issue

Reset issue

Blocking assignment

Timing issue

Important GTKWave Signals to Observe

Always add:

Clk

Rst

Input signals

Output signals

VERY IMPORTANT Debugging Skill

Observe:

When signal changes

Not just:

What signal changes

Timing matters in hardware.

Practice Debugging Example

Broken Counter Code

Module counter(

Input clk,

Input rst,

Output reg [3:0] count

);

Always @(posedge clk)

Begin

If(rst)

Count = 0;

Else

Count = count + 1;

End

Endmodule

Find Problems

Issues:

- Uses blocking assignment

- Reset missing in sensitivity list

Correct Version

Module counter(

Input clk,

Input rst,

Output reg [3:0] count

);

Always @(posedge clk or posedge rst)

Begin

If(rst)

Count <= 0;

Else

Count <= count + 1;

End

Endmodule

Important Interview Questions

- Difference Between = and <=

=	<=

Blocking	Non-blocking

Sequentially executes	Parallel update

Used in combinational	Used in sequential

- Why Use Reset?

To initialize hardware into known state.

- What is x in waveform?

Unknown value.

Caused by:

Uninitialized signal

Multiple drivers

Simulation issue

- What is z?

High impedance.

Usually occurs in:

Tri-state buffers

Floating wires

Part 5 — Debugging Practice Tasks

Task 1

Create:

Broken D Flip-Flop

Then debug it.

Task 2

Remove:

Always #5 clk = ~clk;

Observe waveform.

Task 3

Replace:

<=

With:

=

Observe differences.

Task 4

Forget reset initialization.

Observe:

Xxxx

States.

Mini Challenge

Build:

4-bit Up/Down Counter

Inputs:

Clk

Rst

Mode

Behavior:

Mode=1 → count up

Mode=0 → count down

Hint:

If(mode)

Count <= count + 1;

Else

Count <= count – 1;

End of Week 2 Skills

You now know:

✅ Combinational circuits

✅ Sequential circuits

✅ Flip-flops

✅ Shift registers

✅ Counters

✅ Clock generation

✅ Testbench writing

✅ Waveform debugging

✅ Simulation flow

This is already stronger than many beginners.

Tomorrow — Day 15

Parameterized Modules

You’ll learn:

Reusable RTL design

Generic modules

Scalable counters/adders

Industry-style coding

Very important for professional RTL design.